COMPUTER NETWORKS

LAB

Faculty of Software Engineering SUPERIOR UNIVERSITY LAHORE PAKISTAN

Lab 06

What is Routing?

Routing is the process of sending data from one network to another.

- A router decides the best path to reach the destination.

There are 2 types:

- Static Routing → Admin manually tells the router where to go.

- Dynamic Routing → Routers learn paths automatically using protocols.

What is Dynamic Routing?

Dynamic Routing means routers automatically exchange routes with each other using a routing protocol.

- If a new network is added, routers update their tables themselves.

- It reduces manual work compared to static routing.

Think of it like Google Maps:

- If a road is blocked, Google Maps auto-updates and shows a new route.

- That’s exactly what dynamic routing does inside a network.

Advantages of Dynamic Routing

- Automatic updates (no need to reconfigure every time).

- Scalable → good for large networks.

- Fault-tolerant → reroutes traffic if a link goes down.

Types of Dynamic Routing Protocols

- Distance Vector Protocols

  - Example: RIP (Routing Information Protocol)

  - Use hop count as metric.

  - Simple but not efficient for big networks.

- Link State Protocols

  - Example: OSPF (Open Shortest Path First)

  - Use bandwidth/cost to choose best path.

  - Fast and reliable for large networks.

- Hybrid Protocols

  - Example: EIGRP (Enhanced Interior Gateway Routing Protocol)

  - Combines both methods.

What is RIP?

RIP (Routing Information Protocol) is a dynamic routing protocol.

- It helps routers share information automatically about the networks they know.

- So, instead of an admin manually telling the router every route (static routing), RIP lets routers learn routes from each other.

RIP is like routers gossiping about roads every 30 seconds, and choosing the shortest route (fewest hops) to deliver data.

How does RIP work?

- Each router tells its neighbors:
“Here are the networks I can reach, and how many steps (hops) away they are.”

- Routers keep updating each other every 30 seconds.

- RIP chooses the path with the least number of hops (shortest path).

Hop means one router.
Example:
If data goes from Router A → Router B → Router C, that’s 2 hops.

Real-Life Example

Imagine you’re in a city:

- You want to go from your house to a park.

- You ask your friend (Router A) for directions.

- Your friend says:

  - “Park is 2 streets away through me.”

- Another friend (Router B) says:

  - “Park is 5 streets away through me.”

- You will choose your first friend’s route (because 2 hops < 5 hops).

That’s how RIP decides the best path!

Features of RIP

- Easy to configure (good for beginners).

- Updates every 30 seconds.

- Limit → Maximum 15 hops. If a network is more than 15 hops away, RIP cannot reach it.

- Versions:

  - RIP v1 → Old, doesn’t support subnetting (classful).

  - RIP v2 → New, supports VLSM and subnet masks (classless).